Flame Sensor

Usage: These types of sensors are used for short range fire detection and can be used to monitor projects or as a safety precaution to cut devices off / on.

Range: about 3 feet.

How it works: The flame sensor is very sensitive to IR wavelength at 760 nm ~ 1100 nm light.

Analog output (A0): Real-time output voltage signal on the thermal resistance.

Digital output (D0): When the temperature reaches a certain threshold, the output high and low signal threshold adjustable via potentiometer.

Pins:

VCC...... Positive voltage input: 5v for analog 3.3v for Digital.

A0.......... Analog output

D0......... Digital output

GND..... Ground

The following code maps and reads the analog values given by the flame sensor (0-1024). The stock flame sensor will have the following reaction with this code: If holding a flame within 1.5 feet in front of the sensor; "case 0" will be activated and " ** Close Fire ** " will be sent to the serial monitor. If holding a flame between 1.5 feet and 3 feet in front of the sensor; "case 1" will be activated and " **Distant Fire** " will be sent to the serial monitor. If no flame is detected in front of the sensor; "case 2" will be activated and " No Fire " will be sent to the serial monitor.

const int sensorMin = 0;     // sensor minimum

const int sensorMax = 1024;  // sensor maximum

void setup() {

// initialize serial communication @ 9600 baud:

Serial.begin(9600);

}

void loop() {

// read the sensor on analog A0:

int sensorReading = analogRead(A0);

// map the sensor range (four options):

// ex: 'long int map(long int, long int, long int, long int, long int)'

int range = map(sensorReading, sensorMin, sensorMax, 0, 3);

// range value:

switch (range) {

case 0:    // A fire closer than 1.5 feet away.

Serial.println("** Close Fire **");

break;

case 1:    // A fire between 1-3 feet away.

Serial.println("** Distant Fire **");

break;

case 2:    // No fire detected.

Serial.println("No Fire");

break;

}

delay(1);  // delay between reads

}