# Name: Mohammed AL-rejjal.

# ID : 31801001002.

# Subject: COMPUTER ARCHITECTURE

# DR: Mohammed Ababneh

# The point: Explain the concept of Von Neumann bottle neck concept, show the cause of this phenomena, and how this phenomena has been addressed by different researchers and designers of computer architectures and organizations.

# Date:12/11/2019

The von Neumann architecture—also known as the von Neumann model or Princeton architecture—is a computer architecture based on a 1945 description by the mathematician and physicist John von Neumann and others in the First Draft of a Report on the EDVAC.[1] That document describes a design architecture for an electronic digital computer with these components:

- A processing unit that contains an arithmetic logic unit and processor registers

- A control unit that contains an instruction register and program counter

- Memory that stores data and instructions

- External mass storage

- Input and output mechanisms

The term "von Neumann architecture" has evolved to mean any stored-program computer in which an instruction fetch and a data operation cannot occur at the same time because they share a common bus. This is referred to as the von Neumann bottleneck and often limits the performance of the system

The design of a von Neumann architecture machine is simpler than a Harvard architecture machine—which is also a stored-program system but has one dedicated set of address and data buses for reading and writing to memory, and another set of address and data buses to fetch instructions.

A stored-program digital computer keeps both program instructions and data in read-write, random-access memory (RAM). Stored-program computers were an advancement over the program-controlled computers of the 1940s, such as the Colossus and the ENIAC. Those were programmed by setting switches and inserting patch cables to route data and control signals between various functional units. The vast majority of modern computers use the same memory for both data and program instructions. The von Neumann vs. Harvard distinction applies to the cache architecture, not the main memory (split cache architecture).

On a large scale, the ability to treat instructions as data is what makes assemblers, compilers, linkers, loaders, and other automated programming tools possible. It makes "programs that write programs" possible. This has made a sophisticated self-hosting computing ecosystem flourish around von Neumann architecture machines.

Some high level languages leverage the von Neumann architecture by providing an abstract, machine-independent way to manipulate executable code at runtime (e.g., LISP), or by using runtime information to tune just-in-time compilation (e.g. languages hosted on the Java virtual machine, or languages embedded in web browsers).

On a smaller scale, some repetitive operations such as BITBLT or pixel and vertex shaders can be accelerated on general purpose processors with just-in-time compilation techniques. This is one use of self-modifying code that has remained popular.

The mathematician Alan Turing, who had been alerted to a problem of mathematical logic by the lectures of Max Newman at the University of Cambridge, wrote a paper in 1936 entitled On Computable Numbers, with an Application to the Entscheidungsproblem, which was published in the Proceedings of the London Mathematical Society.[6] In it he described a hypothetical machine he called a universal computing machine, now known as the "Universal Turing machine". The hypothetical machine had an infinite store (memory in today's terminology) that contained both instructions and data. John von Neumann became acquainted with Turing while he was a visiting professor at Cambridge in 1935, and also during Turing's PhD year at the Institute for Advanced Study in Princeton, New Jersey during 1936 – 1937. Whether he knew of Turing's paper of 1936 at that time is not clear.

In 1936, Konrad Zuse also anticipated in two patent applications that machine instructions could be stored in the same storage used for data.

Independently, J. Presper Eckert and John Mauchly, who were developing the ENIAC at the Moore School of Electrical Engineering, at the University of Pennsylvania, wrote about the stored-program concept in December 1943. In planning a new machine, EDVAC, Eckert wrote in January 1944 that they would store data and programs in a new addressable memory device, a mercury metal delay line memory. This was the first time the construction of a practical stored-program machine was proposed. At that time, he and Mauchly were not aware of Turing's work.

Von Neumann was involved in the Manhattan Project at the Los Alamos National Laboratory, which required huge amounts of calculation. This drew him to the ENIAC project, during the summer of 1944. There he joined the ongoing discussions on the design of this stored-program computer, the EDVAC. As part of that group, he wrote up a description titled First Draft of a Report on the EDVAC[1] based on the work of Eckert and Mauchly. It was unfinished when his colleague Herman Goldstine circulated it with only von Neumann's name on it, to the consternation of Eckert and Mauchly. The paper was read by dozens of von Neumann's colleagues in America and Europe, and influenced the next round of computer designs.

Jack Copeland considers that it is "historically inappropriate, to refer to electronic stored-program digital computers as 'von Neumann machines His Los Alamos colleague Stan Frankel said of von Neumann's regard for Turing's ideas:

I know that in or about 1943 or '44 von Neumann was well aware of the fundamental importance of Turing's paper of 1936… Von Neumann introduced me to that paper and at his urging I studied it with care. Many people have acclaimed von Neumann as the "father of the computer" (in a modern sense of the term) but I am sure that he would never have made that mistake himself. He might well be called the midwife, perhaps, but he firmly emphasized to me, and to others I am sure, that the fundamental conception is owing to Turing— in so far as not anticipated by Babbage… Both Turing and von Neumann, of course, also made substantial contributions to the "reduction to practice" of these concepts but I would not regard these as comparable in importance with the introduction and explication of the concept of a computer able to store in its memory its program of activities and of modifying that program in the course of these activities.

At the time that the "First Draft" report was circulated, Turing was producing a report entitled Proposed Electronic Calculator. It described in engineering and programming detail, his idea of a machine he called the Automatic Computing Engine (ACE). He presented this to the Executive Committee of the British National Physical Laboratory on February 19, 1946. Although Turing knew from his wartime experience at Bletchley Park that what he proposed was feasible, the secrecy surrounding Colossus, that was subsequently maintained for several decades, prevented him from saying so. Various successful implementations of the ACE design were produced.

Both von Neumann's and Turing's papers described stored-program computers, but von Neumann's earlier paper achieved greater circulation and the computer architecture it outlined became known as the "von Neumann architecture". In the 1953 publication Faster than Thought: A Symposium on Digital Computing Machines (edited by B. V. Bowden), a section in the chapter on Computers in America reads as follows:

The Machine of the Institute For Advanced Studies, Princeton

In 1945, Professor J. von Neumann, who was then working at the Moore School of Engineering in Philadelphia, where the E.N.I.A.C. had been built, issued on behalf of a group of his co-workers, a report on the logical design of digital computers. The report contained a detailed proposal for the design of the machine that has since become known as the E.D.V.A.C. (electronic discrete variable automatic computer). This machine has only recently been completed in America, but the von Neumann report inspired the construction of the E.D.S.A.C. (electronic delay-storage automatic calculator) in Cambridge (see page 130).

In 1947, Burks, Goldstine and von Neumann published another report that outlined the design of another type of machine (a parallel machine this time) that would be exceedingly fast, capable perhaps of 20,000 operations per second. They pointed out that the outstanding problem in constructing such a machine was the development of suitable memory with instantaneously accessible contents. At first they suggested using a special vacuum tube—called the "Selectron"—which the Princeton Laboratories of RCA had invented. These tubes were expensive and difficult to make, so von Neumann subsequently decided to build a machine based on the Williams memory. This machine—completed in June, 1952 in Princeton—has become popularly known as the Maniac. The design of this machine inspired at least half a dozen machines now being built in America, all known affectionately as "Johniacs."

In the same book, the first two paragraphs of a chapter on ACE read as follows

Automatic Computation at the National Physical Laboratory

One of the most modern digital computers which embodies developments and improvements in the technique of automatic electronic computing was recently demonstrated at the National Physical Laboratory, Teddington, where it has been designed and built by a small team of mathematicians and electronics research engineers on the staff of the Laboratory, assisted by a number of production engineers from the English Electric Company, Limited. The equipment so far erected at the Laboratory is only the pilot model of a much larger installation which will be known as the Automatic Computing Engine, but although comparatively small in bulk and containing only about 800 thermionic valves, as can be judged from Plates XII, XIII and XIV, it is an extremely rapid and versatile calculating machine.

The basic concepts and abstract principles of computation by a machine were formulated by Dr. A. M. Turing, F.R.S., in a paper1. read before the London Mathematical Society in 1936, but work on such machines in Britain was delayed by the war. In 1945, however, an examination of the problems was made at the National Physical Laboratory by Mr. J. R. Womersley, then superintendent of the Mathematics Division of the Laboratory. He was joined by Dr. Turing and a small staff of specialists, and, by 1947, the preliminary planning was sufficiently advanced to warrant the establishment of the special group already mentioned. In April, 1948, the latter became the Electronics Section of the Laboratory, under the charge of Mr. F. M. Colebrook.